SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0704

NAME: Shaik Roshan

REG NO: 192425258

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

    - Determine the network address and broadcast address for each department.

    - Identify the valid host IP address range for each subnet.

    - Calculate the number of usable host addresses available in each subnet.

    - Determine the remaining unused IP address range within the given network block.

ANSWER:

Given Network: 192.168.100.0/24

| Department | Subnet |
| --- | --- |
| School of Computing | /25 |
| School of Electronics | /26 |
| School of Mechanical Engineering | /27 |

## 1. Network Address & Broadcast Address

| Department | Network Address | Broadcast Address |
| --- | --- | --- |
| Computing (/25) | 192.168.100.0 | 192.168.100.127 |
| Electronics (/26) | 192.168.100.128 | 192.168.100.191 |
| Mechanical (/27) | 192.168.100.192 | 192.168.100.223 |

## 2. Valid Host Range

| Department | Host Range |
| --- | --- |
| Computing | 192.168.100.1 – 192.168.100.126 |
| Electronics | 192.168.100.129 – 192.168.100.190 |
| Mechanical | 192.168.100.193 – 192.168.100.222 |

## 3. Number of Usable Hosts

| Department | Hosts |
| --- | --- |
| Computing (/25) | 126 |
| Electronics (/26) | 62 |
| Mechanical (/27) | 30 |

## 4. Remaining Unused IP Range

Allocated till:

192.168.100.223

Remaining:

192.168.100.224 – 192.168.100.255

(32 IP addresses remain.)

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each department.

    - Identify the valid range of host IP addresses.

    - Calculate the number of usable hosts in each subnet.

    - Identify the unused IP address range remaining within the allocated IP block.

# ANSWER:

Given Network: 10.10.0.0/16

| Department | Subnet |
| --- | --- |
| Software Development | /24 |
| Quality Assurance | /25 |
| Human Resources | /26 |
| Executive Management | /27 |

Subnet allocation is done sequentially.

## 1. Network & Broadcast Address

| Department | Network | Broadcast |
| --- | --- | --- |
| Software Development | 10.10.0.0/24 | 10.10.0.255 |
| Quality Assurance | 10.10.1.0/25 | 10.10.1.127 |
| Human Resources | 10.10.1.128/26 | 10.10.1.191 |
| Executive Management | 10.10.1.192/27 | 10.10.1.223 |

## 2. Host Range

| Department | Host Range |
| --- | --- |
| Software Development | 10.10.0.1 – 10.10.0.254 |
| Quality Assurance | 10.10.1.1 – 10.10.1.126 |
| Human Resources | 10.10.1.129 – 10.10.1.190 |
| Executive Management | 10.10.1.193 – 10.10.1.222 |

## 3. Usable Hosts

| Department | Hosts |
| --- | --- |
| Software Development | 254 |
| Quality Assurance | 126 |
| Human Resources | 62 |
| Executive Management | 30 |

## 4. Remaining Unused Range

Used addresses:

10.10.0.0 – 10.10.1.223

Unused begins from:

10.10.1.224 onwards up to 10.10.255.255

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

    - Convert the given IPv6 address into its compressed notation.

    - Expand the compressed address back into its full hexadecimal representation.

    - Identify the network prefix using the /64 prefix length.

    - Calculate the total number of host addresses supported within the subnet.

    - the total number of possible host addresses available within this network.

ANSWER:

IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

## 1. Compressed Notation

2001:db8:abcd::/64

## 2. Expanded Notation

2001:0db8:abcd:0000:0000:0000:0000:0000

## 3. Network Prefix

With /64

Network Prefix:

2001:0db8:abcd:0000::/64

Host Portion:

0000:0000:0000:0000

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

ANSWER:

Routing Table

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

Destination IP

192.168.2.45

## 1. Destination Subnet

192.168.2.0/24

## 2. Matching Routing Table Entry

192.168.2.0/24

## 3. Next Hop / Interface

Forward through the router interface connected to

192.168.2.0/24

## 4. Default Route

If destination does not match any entry:

0.0.0.0/0

(Default Gateway)

- A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

ANSWER:

Administration LAN

192.168.10.0/26

Finance LAN

192.168.10.64/26

Gateways

Administration:

192.168.10.1

Finance:

192.168.10.65

## 1. Network & Broadcast Address

| LAN | Network | Broadcast |
| --- | --- | --- |
| Administration | 192.168.10.0 | 192.168.10.63 |
| Finance | 192.168.10.64 | 192.168.10.127 |

## 2. Valid Host Range

| LAN | Host Range |
| --- | --- |
| Administration | 192.168.10.1 – 192.168.10.62 |
| Finance | 192.168.10.65 – 192.168.10.126 |

## 3. Suggested IP Addresses

### Administration LAN

Computer 1

192.168.10.2

Computer 2

192.168.10.3

Computer 3

192.168.10.4

### Finance LAN

Computer 1

192.168.10.66

Computer 2

192.168.10.67

Computer 3

192.168.10.68

## 4. Default Gateway

| LAN | Default Gateway |
| --- | --- |
| Administration | 192.168.10.1 |
| Finance | 192.168.10.65 |

These answers directly address all the tasks in the uploaded assessment document.

RUBRICS FOR CALCULATION

|  | Scenario Based Assessment |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes wellreasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |